Dynamics and Vibrations – Literature Review

Black: Completed review

Green: In progress

Red: Not Started

Contents

1. General Vibrations	5

Shock Response Spectrum – A Primer	5

Author: J. Edward Alexander	5

Investigation into th Dynamic Response of Cylinder Liners in an IC Engine Based on a Validate Finite-Element Model	9

Authors: Guoxing Li, Fengshou Gu, Tie Wang, Tiantian Yang, and Andrew Ball	9

2. Chaos	10

A Heuristic Method for Identifying Chaos from Frequency Content	10

Authors: R. Wiebe and L. N. Virgin	10

Deterministic Chaos Theory: Basic Concepts	11

The Harmonic Balance Method for Bifucation Analysis of Large-Scale Nonlinear Mechanical Systems	13

Authors: T. Detroux, L. Renson, L. Masset, and G. Kerschen	13

3. Discretization for Vibration	20

Comparison of Numerical Integration Methods in the Linear Dynamic Analysis	20

Authors: Danijela M. Miloradovic, Gordana Bogdanovic, Lozica Ivanovic, Vladminir Geroski, and Marija Rafailovic	20

Physics Based Sound Synthesis – Maarten van Walstijn	21

1. Introduction	21

2. Math Review	21

3. Vibration of Point Masses	21

4. Discretization of the Harmonic Oscillator	21

5. Vibration of Strings and Membranes	21

6. Finite Difference Simulation	21

7. Efficient String Instrument Synthesis	21

8. DSP Elements in MAX-MSP	21

4. Energy Dissipation	22

A Nonlinear Energy Sink with an Energy Harvester: Transient Responses	22

Authors: Daniel Kremer and Kefu Liu	22

Nonlinear Vibration Control and Energy Harvesting of a Beam using a Nonlinear Energy Sink and a Piezoelectric Device	23

Authors: Z. Nili Ahmadabadi, and S. E. Khadem	23

Tuning of Nonlinear Vibration via Topology Variation and its Application in Energy Harvesting	24

Authors: Xuhan Dai, Xiaodan Miao, Linghe Sui, Hailin Zhou, Xiaolin Zhao, and Guifu Ding	24

5. Fourier and Orthogonal Decomposition	25

Limited-memory Adaptive Snapshot Selection for Proper Orthogonal Decomposition	25

Authors: Geoffrey M. Oxberry, Tanya Kostova-Vassilevska, William Arrighi, and Kyle Chand	25

Overview of FFT-Based homogenization Techniques from the Galerkin Point of View	26

Author: Sebastien Brisard	26

Persistent Model Order Reduction for Complex Dynamical Systems using Smooth Orthogonal Decomposition	27

Authors: Shabad Ilbeigi, and David Chelidze	27

The Dynamics of Coherent Structures in the Wall Region of a Turbulent Boundary Layer	28

Authors: Nadine Aubry, Philip Holmes, John L. Lumley, and Emily Stone	28

6. LANL Reports	29

LA-13070 – Damage Identification and Health Monitoring of Structural and Mechanical Systems from Changes in Their Vibration Characteristics: A Literature Review	29

LA-14535 – Nonlinear System Identification for Damage Detection	30

LA-14393 – Structural Health Monitoring Algorithm Comparisons Using Standard Data Sets	31

LA-14489 – Feature Extraction for Structural Dynamics Model Validation	33

7. Nonlinear Damping Systems	37

An Equivalent Method for Optimization of Particle Tuned Mass Damper Based on Experimental Parametric Study	37

Authors: Zheng Lu, Xiaoyi Chen, Ying Zhou	37

Chapter 7 – Nonlinear Vibrations of Axially Moving Beams	38

Author: Li-Qun Chen	38

Investigating the Robustness of Nonlinear Energy Sink in Steady State Dynamics of Linear Beams with Different Boundary Conditions	39

Authors: Masoumeh Parseh, Morteza Dardel, and Mohammad Hassan Ghasemi	39

Nonlinear Dissipative Devices in Structural Vibration Control: A Review	40

Authors: Zheng Lu, Zixin Wang, Ying Zhou, and Xilin Lu	40

Nonlinear Vibration Control of a Cantilever Beam by a Nonlinear Energy Sink	41

Authors: Z. Nili Ahmadabadi and S. E. Khadem	41

Nonlinear Vibration Control of a Tall Structure with Tuned Liquid Column Damper	42

Author: Shueei-Muh Lin	42

Realization of a Strongly Nonlinear Vibration-Mitigation Device Using Elastomeric Bumpers	43

Authors: Jie Luo, Nicholas E. Wierschem, Larry A. Fahnestock, Lawrence A. Bergman, Billie F. Spencer, Mohammad Al-Shudeifat, D. Michael McFarland, D. Dane Quinn, and Alexander F. Vakakis	43

Seismic Control of Power Transmission Tower Using Pounding Tuned Mass Damper	44

Authors: Peng Zhang, Gangbing Song, Hong-Nan Li, and You-Xin Lin	44

Studies of the Performance of Particle Dampers under Dynamic Loads	45

Authors: Zheng Lu, Xilin Lu, and Sami F. Masri	45

Vibration Control of a Power Transmission Tower with Pounding Tuned Mass Damper under Multi-Component Seismic Excitations	46

Authors: Li Tian, Kunjie Rong, Peng Zhang, and Yuping Liu	46

8. Standards	47

ASTM D999-15	47

Standard Test Methods for Vibration Testing of Shipping Containers	47

ASTM D3580-15	48

Standard Test Methods for Vibration (Vertical Linear Motion) Test of Products	48

ASTM D4169-16	49

Standard Practice for Performance Testing of Shipping Containers and Systems	49

ASTM D4728-17	50

Standard Test Method for Random Vibration Testing of Shipping Containers	50

MIL-STD-810F - Part 2 Laboratory Tests	51

514.5 Vibration (pp. 227-273)	51

MIL-STD-2073-1	52

Packaging and Marking Guide for DoD	52

MSFC-STD-3676-A	53

Development of Vibro-Acoustic and Shock Design and Test Criteria	53

NASA-HDBK-7008	54

Spacecraft Dynamic Environments Testing	54

9.	Structural Health Monitoring	55

Development of a Smart-device-based Vibration-measurement System: Effectiveness examination and Application cases to existing structure	55

Authors: Ashish Shrestha, Ji Dang, and Xin Wang	55

# 1. General Vibrations

## Shock Response Spectrum – A Primer

### Author: J. Edward Alexander

A shock is a sudden and violent change in the state of motion of the component parts or particles of a body or medium resulting from the sudden application of a relatively large external force, such as a blow or impact.

SDOF equations of motion (no damping) for base excitation ub(t) :

zt+ω2zt=-ub(t)

Where ω2 is stiffness over mass (k/m). The coordinate z(t) is the relative motion of the SDOF to the base motion; z(t) = x(t) – ub(t). The solution to this equation of motion is obtained from Duhamel’s Integral yielding:

zt=-1ω0tub(τ)sinωt-τdτ

The absolute acceleration of the mass is proportional to the relative displacement. After some manipulation, the absolute acceleration is:

xt=ω0tub(τ)sinωt-τdτ

Shock Response Spectrum

Shock Response Spectrum (SRS) analysis is the maximum response of a series of single degree of freedom (SDOF) systems [having the] same damping to a given transient signal. An SRS transforms the acceleration-time history from the time domain to the maximum response of a SDOF system in the frequency domain.

The SRS of a SDOF system can be determined by determining the maximum absolute acceleration for each frequency:

SA=x(t)max

Substituting this into the solution for the absolute acceleration results in the following SRS equations:

SA=x(t)max=ω0tub(τ)sinωt-τdτmax (No Damping)

SA=ω0tub(τ)e-ξω(t-τ)sinωt-τdτmax (With Damping)

Discussion of different type of shock events is provided. Earthquake maximum spectral accelerations can be around 3.5g with durations of 10 seconds, while naval shocks can be 30g with durations of 100 milliseconds, and pyrotechnic shocks can be nearly 2500g with durations of 10s of milliseconds.

Discussion of events characterized by SRS and Design Spectra

The shock response spectra can be plotted as a four coordinate (tripartite) grid with velocity vs. system period in time. The acceleration and displacement histories can be viewed on the grids that are 45° clockwise and counter-clockwise, respectively, relative to the velocity grid.

For the design of structures subjected to shock loading, three historical methods are discussed primarily from earthquake data: Biot, Housner, and Newmark design spectra. For relatively low and high frequency base motion, the design spectra is equivalent to the base motion. For intermediate frequencies, there is a region of amplification that of course depend on the properties of the structure.

SRS Analysis of Linear MDOF Structures

For linear multiple degree of freedom (MDOR) structures, the maximum dynamic system response can be approximated by the use of mode superposition. This technique transforms the system dynamics for physical to modal coordinates in which each mode behaves as a single degree of freedom. Modal transformation requires eigenvalue extraction to find the eigenvalues and eigenvectors. The SRS can be applied to the modal system by considering the participation factors for each mode frequency. If the system contains nonlinearities, then eigenvalue decomposition is not possible!

MDOF equations of motion for free vibration and base excitation:

Mzt+Kzt=0

Mzt+Kzt=-M1ub(t)

If simple harmonic motion of the following form is assumed:

zi=ϕisinωit

When substituted into the free vibration equation, the eigenvalue problem results in:

ωi2Mϕi=Kϕi

The eigenvalues are given by ωi2 and eigenvectors by {ϕ}i which can be assembled into an eigenvector matrix:

ϕ=ϕ1ϕ2ϕ3…ϕn

The eigenvector matrix is used to transform relative physical coordinates, z(t), into modal coordinates, x(t):

z(t)=ϕx(t)

Substituting [ϕ]x(t) into the forced vibration equation (base excitation) and pre-multiplying by [ϕ]T yields:

ϕTMϕxt+ϕTKϕxt=-ϕTM1ub(t)

The matrices [ϕ]T[M][ϕ] and [ϕ]T[K][ϕ] are diagonal matrices due to orthogonality properties of normal modes; thanks to pre-multiplication by [ϕ]T. The resulting equation for the ith mode of vibration is:

xit+ϕiTKϕiϕiTMϕixit=-ϕiTM1ϕiTMϕiub(t)

The coefficient of xi(t) is the squared circular frequency of the ith mode and the coefficient of the base acceleration is the participation factor given by:

ωi2=ϕiTKϕiϕiTMϕi

Pi=ϕiTM1ϕiTMϕi

Which yields:

xit+ωi2xit=-Piub(t)

This equation is nearly identical to the SDOF equation of motion. The transformation back to physical coordinate accelerations is given by:

z(t)=i=1nϕixi(t)

The absolute physical coordinate acceleration can thus be expressed as:

u(t)=i=1nϕiPixi(t)

The maximum response can be expressed using the equation below. The equal sign is replaced by an inequality since the maximum modal coordinate acceleration will not occur at the same time. The maximum acceleration can be replaced by the spectral acceleration which yields:

umax≤i=1nϕiPiximax=i=1nϕiPiSAi

Since all of the modal maximum will not occur at the same time, an estimate of the max acceleration of the jth mass is typically obtained using a root sum square approach given by:

uj max≅i=1nϕiPiSAi2

This equation shows that modal superposition in conjunction with the SRS can be used to provide an estimate of the maximum dynamic response of a linear system without solving the transient MDOF equations of motion.

## Investigation into the Dynamic Response of Cylinder Liners in an IC Engine Based on a Validated Finite-Element Model

### Authors: Guoxing Li, Fengshou Gu, Tie Wang, Tiantian Yang, and Andrew Ball

# 2. Chaos

## A Heuristic Method for Identifying Chaos from Frequency Content

### Authors: R. Wiebe and L. N. Virgin

Deterministic Chaos Theory: Basic Concepts
Authors: Mauro Cattani, Ibere Luiz Caldas, Silvio Luiz de Souza, Kelly Cristiane, Iarosz

Revista Brasileira de Ensino de Fisica (2017)

This paper is intended to introduce the reader to the study and discussion of chaotic systems, especially from a mathematical basis. In particular is the discussion of deterministic chaos theory which results from deterministic nonlinear ordinary differential equations.

A quote from Poincare: “If we knew exactly the law of nature and the situation of the universe at the initial moment, we could predict exactly the situation of that same universe at a succeeding moment. But even if it were the case that the natural laws had no longer any secret for us, we could still only know the initial situation approximately. If it enabled us to predict the succeeding situation with the same approximation, that is all we require, and we should say that the phenomenon had been predicted, that it is governed by laws. But it is not always so: it may happen that small differences in the initial conditions produce very great ones in the final phenomena. A small error in the former will produce an enormous error in the latter. Prediction becomes impossible, and we have the fortuitous phenomenon.”

What is deterministic chaos?

We have also learned that deterministic chaos is always associated with nonlinear systems; nonlinearity is a necessary condition for chaos, but not a sufficient one.

Different from deterministic chaos, random (or stochastic) processes are composed of random variables are represent the evolution of some system of random values over time. There is a probabilistic evolution of the initial states.

Differential equations can be integrated when the following features are manifested: (a) existence of enough number of conserved quantities, (b) existence of an algebraic geometry, and (c) ability to give explicit solutions.

Tools to visualize chaos

Poincare sections are a tool to analyze behavior of the system. Simple curves may have analytic solutions, while complicated/irregular curves may represent chaos. Aperiodic motion is a characteristic of deterministic chaos.

“In chaotic motion, nearby trajectories in phase space are continually diverging from one another following the attractor… due to these attractors, named strange or chaotic attractors, the motions in phase space are necessarily bounded… Amplifying a sub line we would see another set of sub lines and so on… verifying that the strange attractors are fractals.”

Lyapunov exponents

Lyapunove created a characteristic exponent to quantify the sensitive dependence of chaotic behavior on initial conditions which give valuable information about the stability of dynamic systems. These help determine the minimum requirements for a differential system to become chaotic.

## The Harmonic Balance Method for Bifucation Analysis of Large-Scale Nonlinear Mechanical Systems

### Authors: T. Detroux, L. Renson, L. Masset, and G. Kerschen

Computational Methods Applied Mechanical Engineering (2015)

OVERALL: This paper was rather complicated and may only work for systems with periodic excitation. Definitely worth considering if there is an example in which harmonic balance can be applied for our situation, such as for explicit FEA in abaqus. The finite element geometry in their numerical model was also very simplified, so scaling up to the geometry we are interested in would be very different.

1. Introduction

This paper discusses the harmonic balance (HB) method that has been widely used in literature for analyzing the periodic solutions of nonlinear mechanical systems and applies it to bifurcation analysis.

“Time-domain methods, which deal with the resolution of a boundary value problem, have proven effective for low-dimensional problems. When applied to larger systems, their computational burden can become substantial.”

So if we want to use explicit analysis, is there a way to still extract useful information?

Harmonic Balance (HB) is the most widely used method in the frequency domain and is known as the Fourier-Galerkin method. Several improvements to HB are mentioned by the author. A particular improvement is described as the alternating frequency-time (AFT) domain method that incrementally evaluates the nonlinear terms of the equations in the time domain “where their analytical expression is known” and then evaluates equations in the frequency domain.

Other examples are cited from literature in electrical engineering, mechanical engineering, and fluid dynamics. Of note, “the HB method has been applied to realistic examples including bladed disks, bolted joints, rotor/stator contacts, vibro-impact systems, geometrically nonlinear beams and plates … and complete vehicles and full-scale aircraft…. Comparisons between experiments and the results of HB simulations gave further evidence of the accuracy of the method.”

The main contribution of this paper is to adapt classical tools for bifurcation analysis in codimension-2 parameter space to the HB formalism. They target large-scale mechanical system with localized nonlinearities.

2. Harmonic Balance for Periodic Solutions

2.1 Formulation of the dynamics in the frequency domain

Nonautonomous nonlinear dynamical systems with n degrees of freedom governed by:

Mx+Cx+Kx+fnlx,x=fext(ω,t)

Where M, C, and K are the mass, damping, and stiffness matrices. Vectors x, fnl, and fext represent the displacements, the nonlinear forces, and the periodic external forces that are considered to be harmonic with frequency ω.

The periodic signals x(t) and f(x,xdot,ω,t)=fext-fnl are approximated by Fourier Series truncated to the NHth  harmonic:

xt=c0x2+k=1NHskxsinkωtν+ckxcoskωtν

ft=c0x2+k=1NHskfsinkωtν+skfcoskωtν

Where sk and ck represent the vectors of the Fourier coefficients related to the sine and cosine terms respectively, and the integer ν accounts for subharmonics of the excitations frequency ω. Fourier coefficients f(t), ckf, and skf, depend on the Fourier coefficients of the displacements x(t), ckx, and skx, which represent the new unknowns of the problem. These coefficients are gather into the (2NH+1)*n by 1 vectors.

z=c0xTs1xTs1xT⋯sNHxTcNHxTT

b=c0fTs1fTs1fT⋯sNHfTcNHfTT

The displacements and the forces are recast into a more compact form

xt=Q(t)⊗Inz

ft=Q(t)⊗Inb

Where ⊗ and In stand for the Kronecker tensor product and the identity matrix of size n, respectively, and Q(t) is a vector containing sine and cosine series:

Qt=12sinωtνcosωtν⋯sinωNHtνcosωNHtν

Velocities and accelerations can also be defined using the Fourier series, with

xt=Q(t)⊗Inz=Q(t)∇⊗Inz

xt=Q(t)⊗Inz=Q(t)∇2⊗Inz

Where

∇=0⋱∇k⋱∇NH,       ∇2=∇∇=0⋱∇k2⋱∇NH2

With

∇k=0-kωνkων0,    ∇k2=-kων200-kων2

Where some rearranging and skipped steps gives

Q(t)∇2⊗Mz+Q(t)∇⊗Cz+Q(t)⊗Kz=Q(t)⊗Inb

In order to remove the time dependency and to obtain an expression relating the different Fourier coefficients, a Galerkin procedure projects the previous equation on the orthogonal trigonometric basis Q(t)

2T0TQ(t)TQ(t)dt∇2⊗Mz+2T0TQ(t)TQ(t)dt∇⊗Cz+2T0TQ(t)TQ(t)dt⊗Kz=2T0TQ(t)TQ(t)dt⊗Inb

Where T is the period of the external force. Considering that

2T0TQtTQtdt=I2NH+1

The equations of motion expressed in the frequency domain are eventually obtained

∇2⊗Mz+∇⊗Cz+I2NH+1⊗Kz=I2NH+1⊗Inb

Or, in a more compact form,

hz,ω≡Aωz-bz=0

where A is the (2NH+1)*n by (2NH+1)*n matrix describing the linear dynamics

A=∇2⊗M+∇⊗C+I2NH+1⊗K

A=KK-ων2M-ωνCωνCK-ων2M⋱K-NHων2M-NHωνCNHωνCK-NHων2M

2.2 Analytical expression of the nonlinear terms and of the Jacobian matrix of the system

The second to last equation (h(z,ω)) is nonlinear and has to be solved iteratively (with a Newton-Raphson procedure or hybrid Powell nonlinear solver). At each iteration, an evaluation of b and of ∂h/∂z is needed. Alternating frequency time-domain (AFT) can be used to compute b:

zFFT-1fx,x,ω,tFFTb(z)

Calculating the Jacobian matrix of this system can be computationally demanding, but an alternative method was proposed by Hwang called trigonometric collocation. Denoting N as the number of time samples of a discretized period of oscillation, one defines vectors xtild and ftild containing the concatenated nN time samples of the displacements and the forces, respectively, for all degrees of freedom.

x=x1t1…x1tN…xnt1…xntNT

f=f1t1…f1tN…fnt1…fntNT

The inverse Fourier transform can then be written as a linear operation:

x=Γωz

With the nN by (2NH+1)*n sparse operator

Γω=In⊗1212⋮12In⊗sinωt1νsinωt2ν⋮sinωtNνIn⊗cosωt1νcosωt2ν⋮cosωtNν…In⊗sinNHωt1νsinNHωt2ν⋮sinNHωtNνIn⊗cosNHωt1νcosNHωt2ν⋮cosNHωtNν

The direct Fourier transformation is written

z=Γω+x

Where + stands for the Moore-Penrose pseudoinverse:

Γ+=ΓTΓΓT-1

The Fourier coefficients of the external and nonlinear forces are simply obtained by transforming the signals in the time domain back to the frequency domain:

b(z)=Γω+f

The Jacobian matrix of the h(z,ω) expression with respect to the Fourier coefficients, z, denoted as hz, can be computed as:

hz=∂h∂z=A-∂b∂z=A-∂b∂f∂f∂x∂x∂z=A-Γ+∂f∂xΓ

In general, the derivatives of the forces with respect to the displacements in the time domain can be expressed analytically, which leads to a very effective computation of the Jacobian matrix. The derivatives have to be evaluated only for the nonlinear degrees of freedom.

2.3 Continuation procedure

Usually, the frequency response is to be computed in a range of frequencies, rather than for a single frequency ω. In this paper, a continuation procedure based on tangent predictions and Moore-Penrose corrections, as in the software “MATCOT”, is coupled to HB. The search for a tangent vector t(i) at an iteration point (z(i), ω(i)) along the branch reads

hzhωt(i-1)Tt(i)=01

Where hω stands for the derivative of h with respect to ω

hω=∂h∂ω=∂A∂ωz

The weird second-to-last equation prevents the continuation procedure from turn back. For the first iteration of the procedure, the sum of the components of the tangent is imposed to be equal to one.

The correction stage is based on Newton’s method. Introducing new optimization variable v(i,j) initialized as v(i,1)=t(i) and y(i,j)=[z(i,j) ω(i,j)]T, the Newton iterations are constructed as follows:

y(i,j+1)=y(i,j)-Gy-1(y(i,j),v(i,j))G(y(i,j),v(i,j))

v(i,j+1)=v(i,j)-Gy-1(y(i,j),v(i,j))R(y(i,j),v(i,j))

With

Gy,v=h(y)0,     Gyy,v=hz(y)hω(y)vT,     Ry,v=hz(y)hω(y)v0

3. Harmonic Balance for bifurcation detection and tracking

3.1 Stability analysis

The continuation procedure in 2.3 does not indicate if a periodic solution is stable or not. For time-domain methods, the shooting technique can be used. For frequency-domain techniques, Hill’s method cane be used and has bee shown to give accurate results for a reasonable number of harmonics NH and to be effective for large systems.

They go on to discuss how to apply Hill’s method in the HB math stuff.

3.2 Detection of bifurcations

To detect bifurcations, test functions φ are evaluated at each iteration of the numerical continuation process. The roots of the test function indicate the presence of bifurcations. A fold bifurcation is found when the iωth component of the tangent prediction related to the active parameter ω changes signs.

Branch point (BP) bifurcations can be detected using the test function for fold bifurcations but with a more specific test function and chunk of math.

The bifurcations they are interested in most are known as Neimark-Sacker (NS) bifurcations. NS bifurcations are detected when a pair of Floquet exponents crosses the imaginary axis as a pair of complex conjugates. Insert math.

3.3 Tracking of bifurcations

“Once a bifurcation is detected, it can be tracked with respect to an additional parameters.”

4. Bifurcation analysis of a satellite structure

The method proposed in this paper is demonstrated on an experimental setup of a small satellite. A reduced finite element analysis was conducted to compare to experiments and test the method. In total, the reduce-order model only contained 37 degrees of freedom.

The overall frequency response computed using HB provides a very accurate estimation of the envelope of the swept-sine response. With some complicated plots, they were also able to show that the numerical system detected an NS bifurcation and lost stability.

This analysis was able to also provide some engineering design information. In [a certain case of parameters and stuff] they were able to determine a value of damping for a component that completely eliminated certain types of bifurcation.

They were also able to show that convergence was achieved for relatively few numbers of harmonics with less than 1% error for NH of 5.

# 3. Discretization for Vibration

## Comparison of Numerical Integration Methods in the Linear Dynamic Analysis

### Authors: Danijela M. Miloradovic, Gordana Bogdanovic, Lozica Ivanovic, Vladminir Geroski, and Marija Rafailovic

## Physics Based Sound Synthesis – Maarten van Walstijn

### 1. Introduction

### 2. Math Review

### 3. Vibration of Point Masses

### 4. Discretization of the Harmonic Oscillator

### 5. Vibration of Strings and Membranes

### 6. Finite Difference Simulation

### 7. Efficient String Instrument Synthesis

### 8. DSP Elements in MAX-MSP

# 4. Energy Dissipation

## A Nonlinear Energy Sink with an Energy Harvester: Transient Responses

### Authors: Daniel Kremer and Kefu Liu

## Nonlinear Vibration Control and Energy Harvesting of a Beam using a Nonlinear Energy Sink and a Piezoelectric Device

### Authors: Z. Nili Ahmadabadi, and S. E. Khadem

## Tuning of Nonlinear Vibration via Topology Variation and its Application in Energy Harvesting

### Authors: Xuhan Dai, Xiaodan Miao, Linghe Sui, Hailin Zhou, Xiaolin Zhao, and Guifu Ding

# 5. Fourier and Orthogonal Decomposition

## Limited-memory Adaptive Snapshot Selection for Proper Orthogonal Decomposition

### Authors: Geoffrey M. Oxberry, Tanya Kostova-Vassilevska, William Arrighi, and Kyle Chand

## Overview of FFT-Based homogenization Techniques from the Galerkin Point of View

### Author: Sebastien Brisard

## Persistent Model Order Reduction for Complex Dynamical Systems using Smooth Orthogonal Decomposition

### Authors: Shabad Ilbeigi, and David Chelidze

## The Dynamics of Coherent Structures in the Wall Region of a Turbulent Boundary Layer

### Authors: Nadine Aubry, Philip Holmes, John L. Lumley, and Emily Stone

Journal of Fluid Mechanics – 1988, Vol. 192

# 6. LANL Reports

## LA-13070 – Damage Identification and Health Monitoring of Structural and Mechanical Systems from Changes in Their Vibration Characteristics: A Literature Review

## LA-14535 – Nonlinear System Identification for Damage Detection

## LA-14393 – Structural Health Monitoring Algorithm Comparisons Using Standard Data Sets

- Introduction

- Experimental Procedure

- Numerical Simulation

- Features Extraction

  - Basic Statistics

    - First Four Statistical Moments of the Measured Data

“In statistics, the data from a random process is said to be stationary and ergodic when the moments and joint moments are time invariant and they do not change when computed over different time historyies.”

The kth central moment about the mean, mk, for a sample x1, … , xn with n observations from a random process X is defined below, where x bar is the mean. Statistics for mean, standard deviation, skewness, and kurtosis can be calculated in this case for k values of 1, 2, 3, and 4, respectively.

mk=1ni=1nxi-xk

    - Probability Density Distribution

A normal probability plot will be linear when the data comes from a normal distribution. If the data are related to another probability distribution, they will introduce curvature in the plot. In a normal probability plot, all points should fall close to a best fit line if the data are normally distributed. For data with nonlinearities, the assumption of normality is not justified and curvature in the probability plot will be introduced, especially in the tails.

The density distribution function is shown below.

fx=1nhi=1nKx-xih

The variable h is the bandwidth and K is the kernel function. The kernel function, K, can be defined as a standard Gaussian function with zero mean and unit variance using the following function.

Kx=12πe-12x2

  - Time-Series Analysis

    - Autoregressive Model

      - Model Identification

      - Model Estimation

    - Principal Component Analysis of Autoregressive Parameters

    - Correlation Coefficient Using Time Domain Data

  - Frequency Domain Analysis

    - Modal Parameters

    - Correlation Coefficients Using Frequency Response Functions

  - Time-Frequency Analysis

    - Short-Time Fourier Transform

    - Wavelet Transform

    - Holder Exponent

- Classification Algorithms

  - Statistical Process Control

    - Statistical Basis of the Shewhart X-bar Control Charts

    - Control Charts with Autocorrelated Process Data

    - Control Chart Analysis

    - Conclusions

  - Cluster Analysis

  - Factor Analysis

- Data Normalization for Feature Classification

- Conclusions and Final Remarks

## LA-14489 – Feature Extraction for Structural Dynamics Model Validation

- Introduction

- Features for Model Validations and Uncertainty Quantification

- Experimental Data Acquisition

- Numerical Model and Calculation

- Features for Structural Dynamics Model Validation

Some of the features used for various types of structural dynamics analyses are summarized below:

Stationary and Gaussian vibrations: modal parameters, frequency spectrum (power spectrum, transfer function), waveform (basic signal statistics, normal probability plot).

Transient dynamics and mechanical shock response: waveform (decrement and exponential decay measures), temporal moments, shock response spectrum.

General-purpose time-series analysis: Autoregressive (AR), autoregressive moving average (ARMA), autoregressive with exogeneous input (ARX), autoregressive and autoregressive with exogenous input (AR-ARX) models, time-frequency transforms, wavelet transform, principal component decomposition.

Unstable, chaotic, multiple-scale dynamics: Holder exponent, state-space maps, time-frequency and higher-order transforms, symmetric dot pattern, fractal analysis.

  - Modal Properties

Modal parameters (i.e. resonant frequencies, modal damping ratios, and mode shapes) can be extracted from linear dynamic systems.

For numerical systems (not FEA), the resonant frequencies and mode shapes are derived from the calculated eigenvalues and eigenvectors of the mass [M] and stiffness [K] matrices. The damping matrix is often neglected in these calculations. The damping ratios can then be found by finding best-fit values using regression.

For experimental systems, the parameters are typically determined from the frequency response function (FRF) or the impulse response functions estimated from measured input and output data. The peaks in the FRF indicate resonant frequencies, and the damping ratios can be derived from the width of the FRF around each peak. Relative amplitude and phase information at the peaks of the FRF can be used to estimate the mode shapes.

In this study (LA-14489), the parameters were estimated by a rational-fraction polynomial curve-fitting method using Forsythe Orthogonal Polynomials; this algorithm is available in LANL’s SHMTools software (SHMTools website).

Note: Since this study only used a 4 degree-of-freedom numerical model, only 4 modes of vibration can be calculated.

  - Waveform Analysis

Features from the waveform analysis can directly compare the time-histories in a quantifiable manner. All of these techniques can be applied to the responses from both linear and nonlinear systems as well as stationary and nonstationary processes.

    - Basic Statistics

| Peak Amplitude: Xpeak | Xpeak=maxxi |
| --- | --- |
| Mean: µx | µx=1Ni=1Nxi |
| Root Mean Square: Xrms | Xrms=1Ni=1Nxi2 |
| Standard Deviation: σx | σx=1Ni=1Nxi-µx2 |
| Skewness: Sx | Sx=1σx31Ni=1Nxi-µx3 |
| Kurtosis: Kx | Kx=1σx41Ni=1Nxi-µx4 |

    - Normal Probability Plot and Probability Density Distribution

See section 4.1.2 in LA-14393.

    - Crest Factor and K-Factor

The crest factor, Xcf, and K-factor, Xkf, are used to diagnose the deviation from sinusoidal response in a harmonic signal.

The crest factor has a theoretical value of 1.414 for a complete sine wave.

Xcf=XpeakXrms

The K-factor represents the global amplitude level of the signal.

Xkf=XpeakXrms

    - 10% Duration and Exponential Decaying Function Fitting

    - Temporal Moments

These are global features! Not well suited to investigate characteristics of events that are local in time such as when impacts occur.

| Energy: E | E=M0 |
| --- | --- |
| Central time: T | T=M1M0 |
| RMS duration: D | D2=M2(T)M0 |
| Central skewness: S | S3=M3(T)M0 |
| Central kurtosis: K | K4=M4(T)M0 |

    - Autocorrelation Function and Correlation Coefficient

Correlation is a statistical technique used in signal processing for analyzing the relationship between two or more signals, or a signal with itself at different time delays, τ. For a stationary process, cross correlation, Rxy, will be in the range of [-1,1] where 1 is perfect correlation and -1 is anti-correlation.

Rxyτ=Ext-μxyt+τ-μyσxσy

Self-correlation, Rxx, can be used to determine the period of time in which a signal is periodic.

Rxxτ=Ext-μxxt+τ-μxσx2

  - Frequency Domain Analysis

    - Frequency Response Function and Coherence Function

    - Power Spectral Density

    - Response Spectra

    - Cepstrum

    - Holder Exponent

  - Time Series

    - Autoregressive Model

    - Autoregressive with Exogenous Input Model

    - AR-ARX Model

- Example of Model Validation using Extracted Features

- Conclusions

## LA-UR-11-05305 – Model Calibration and Validation for Material Damping Using Finite Element Analyses

Damping is a phenomenon observed for all physical systems and is characterized by the dissipation of energy from the system. Damping mechanisms may be classified as external (drag forces, friction) or internal (material damping, thermoelasticity). In some numerical models, damping can be ignored and still produce sufficiently accurate results, especially for quasi-static or short duration events. “In some cases, most notably long-term transient or high-impact plastic deformations, the omission of damping could yield very inaccurate results.”

This study investigate three damping models in order to choose, calibrate, and validate a dynamic FE model. These include mass-proportional, stiffness-proportional, and viscoelastic damping.

Both mass-proportional and stiffness-proportional damping are classified as Rayleigh damping. Rayleigh damping assumes that the damping matrix is a linear combination of the mass and stiffness matrices. The following two equations show that α scales mass contribution and β scales the stiffness contribution. Following modal decomposition, the second equation shows the expression for the ith mode’s damping parameter. The second equation shows that α (mass-proportional damping) affects lower frequency modes while β (stiffness-proportional damping) affects higher frequency modes.

C=αM+βK

ξi=α2ωi+βωi2

The third type of damping used in this study is viscoelastic in which a standard linear solid (SLS) model was chosen. This model is capable of modeling free and forced vibrations, most notably for aluminum.

This study investigated the proposed damping models applicability by comparing to test data collected on an experimental aluminum cantilever beam. An initial displacement as applied before releasing the beam to oscillate. Acceleration and displacement of the tip were measured using an accelerometer and laser displacement sensor, respectively.

Overall, this study was unable to validate an FE model. Mass-proportional and viscoelastic damping were able to match displacements reasonably, but not accelerations. Structural-proportional damping damped out high frequencies too quickly and was computationally expensive.

# 7. Nonlinear Damping Systems

## An Equivalent Method for Optimization of Particle Tuned Mass Damper Based on Experimental Parametric Study

### Authors: Zheng Lu, Xiaoyi Chen, Ying Zhou

## Chapter 7 – Nonlinear Vibrations of Axially Moving Beams

### Author: Li-Qun Chen

## Investigating the Robustness of Nonlinear Energy Sink in Steady State Dynamics of Linear Beams with Different Boundary Conditions

### Authors: Masoumeh Parseh, Morteza Dardel, and Mohammad Hassan Ghasemi

Commun Nonlinear Science Numerical Simulations (2015)

## Nonlinear Dissipative Devices in Structural Vibration Control: A Review

### Authors: Zheng Lu, Zixin Wang, Ying Zhou, and Xilin Lu

Journal of Sound and Vibration – (2018) Vol. 423

## Nonlinear Vibration Control of a Cantilever Beam by a Nonlinear Energy Sink

### Authors: Z. Nili Ahmadabadi and S. E. Khadem

Mechanism and Machine Theory (2012)

## Nonlinear Vibration Control of a Tall Structure with Tuned Liquid Column Damper

### Author: Shueei-Muh Lin

Mechanics of Advanced Materials and Structures

## Realization of a Strongly Nonlinear Vibration-Mitigation Device Using Elastomeric Bumpers

### Authors: Jie Luo, Nicholas E. Wierschem, Larry A. Fahnestock, Lawrence A. Bergman, Billie F. Spencer, Mohammad Al-Shudeifat, D. Michael McFarland, D. Dane Quinn, and Alexander F. Vakakis

ASCE Journal of Engineering Mechanics (2014)

## Seismic Control of Power Transmission Tower Using Pounding Tuned Mass Damper

### Authors: Peng Zhang, Gangbing Song, Hong-Nan Li, and You-Xin Lin

## Studies of the Performance of Particle Dampers under Dynamic Loads

### Authors: Zheng Lu, Xilin Lu, and Sami F. Masri

Journal of Sound and Vibration (2010)

## Vibration Control of a Power Transmission Tower with Pounding Tuned Mass Damper under Multi-Component Seismic Excitations

### Authors: Li Tian, Kunjie Rong, Peng Zhang, and Yuping Liu

Applied Sciences – (2017) Vol. 477

# 8. Standards

## ASTM D999-15

### Standard Test Methods for Vibration Testing of Shipping Containers

“1.1 This test method covers the random vibration testing of filled shipping units. Such tests may be used to assess the performance of a container with its interior packing and means of closure in terms of its ruggedness and the protection that it provides the contents when subjected to random vibration inputs.”

“4.2 Resonance buildups during random vibration tests are less intense than during sinusoidal resonance dwell or sweep tests. Therefore, unrealistic fatigue damage due to resonance buildup is minimized.”

“4.4 There is no direct equivalence between random vibration tests and sinusoidal vibration tests. Equivalent tests between sine and random, in a general sense, are difficult to establish due to nonlinearities, damping, and product response characteristics.”

Four test methods: A1, A2, B, and C. A1 and A2 are repetitive shock tests where A1 is for vertical motion and A2 is for rotary motion. A1 and A2 are conducted on individual containers that are transported unrestrained on the bed of a vehicle. Methods B and C are resonance tests, where B is for a single container and C is for a palletized package. All four methods require a vibration machine. Methods A1, B, and C require motion to be sinusoidal in the vertical direction, while A2 is rotary in the horizontal plane.

Resonance tests (B and C) are conducted by finding the resonant frequencies by either a sine sweep or a random vibration input.

Sine Sweep: the vibration device is adjusted to produce a constant specified acceleration amplitude over a specified range of frequencies (see D4169 for specifications). Starting at the lowest frequency, sweep upward at a logarithmic rate of 0.5-1.0 octaves per minute to the upper frequency and back down. Repeat the sweep twice and take note of resonant frequencies using either auditorily, with a visual device such as a stroboscope, or with accelerometers. (D4169 would require that once resonant frequencies are known, the device would be tested at those specific frequencies of the greatest response for specified dwell times.)

Random Vibration Input:

This method may find resonance frequency/frequencies faster than a sine sweep where a wide frequency input band excites all of the natural frequencies of a packaged product simultaneously. References ASTM D3580. Frequency ranges from 3 to 100 Hz at a minimum power spectral density of 0.005 g2/Hz. The method requires instrumentation of the vibration platform and package to be sure they match. Similarly to the sine sweep, when used in conjunction with D4169, once resonant peaks are determinant, the test is continued at those frequencies for specified dwell times Section 10.10 of D3580 recommends the optional sinusoidal dwell test for found frequencies.

## ASTM D3580-15

### Standard Test Methods for Vibration (Vertical Linear Motion) Test of Products

## ASTM D4169-16

### Standard Practice for Performance Testing of Shipping Containers and Systems

“1.1 This practice provides a uniform basis of evaluating, in a laboratory, the ability of shipping units to withstand the distribution environment. This is accomplished by subjecting them to a test plan consisting of a sequence of anticipated hazard elements encountered in various distribution cycles. This practice is not intended to supplant material specifications or existing pre-shipment test procedures.”

“1.3 The suitability of this practice for use with hazardous materials has not been determined.”

Distribution Cycles (DC) are defined that categorize the potential hazards that may occur for a specific mode of shipping. Prescribed performance tests are given for each DC 1-18. These DCs cover a variety of shipments such as manual handling, truck, rail, air, storage in a warehouse, and combinations thereof.

Procedure:

- Define Shipping Unit

- Establish Assurance Level – assurance level I is most intense and conservative, level III is standard, and level II is in between

- Determine Acceptance Criteria – product is damage-free, intact, or both.

- Select Distribution Cycle (DC) – described above

- Write Test Plan

- Select Samples for Test – representative samples of complete shipping units including actual contents

- Condition Samples – in general, tests are conducted at 23±1°C unless otherwise necessary

- Perform Tests – perform tests as directed in reference ASTM standards

- Evaluate Results – determine if the shipping units meet the acceptance criteria

- Document Test Results

- Monitor Shipments

For certain distribution cycles, vehicle vibration tests are prescribed. Vibration tests are conducted in either of two ways: random test option (ASTM D4728) or sine test option (D999) method B or C. Depending on the distribution cycle and assurance level, Power Spectral Density (PSD) levels are described in tables 2-4 as inputs to the vibration device. For resonant tests, frequency ranges are recommended from 3 to 100 Hz with different amplitudes and resonant peak dwell times depending on assurance level.

For DC 18, the military standard practice for military packing (MIL-STD-2073-1) covers how to properly prepare a package to ensure that it is not degraded during shipment and storage.

For DC 18, Annex A1 describes required test methods for different load conditions. Loose load vibration is tested according to test method D999. Vehicle vibration is conducted according MIL-STD-810F in Part Two (laboratory tests) section “514.5 Vibration.” In general, test method ASTM D4728 is recommended for random vibration testing.

## ASTM D4728-17

### Standard Test Method for Random Vibration Testing of Shipping Containers

This document is very similar to other test methods described (ASTM D999 & D3580) and is used to accompany the random test option of vibration tests prescribed for certain distribution cycles of ASTM D4169.

A vibration test system is required that can produce single axis vibration inputs at controlled levels of continuously variable amplitude through a specific range of frequencies. A “Closed Loop-Automatic Equalization” controller is required that allows the user to enter desired Power Spectral Density (PSD) data by automatically generating equalized vibration data and compensates for specimen vibration using a feedback system. Random vibration should follow a Gaussian distribution to maintain “randomness” but statistically match the desired inputs.

The root-mean-square (RMS) is defined as a key feature of the PSD. In particular, ASTM D4169 defines PSD profiles for certain distribution cycles and assurance levels as coordinates of a particular frequency at a given PSD level (in G2/Hz).

According to the D4169 update rationale, “the root mean square acceleration (Grms) is the square root of the area under the acceleration spectral density curve in the frequency domain. The Grms values is used to express the overall energy of a particular random vibration event and is a statistical value used in vibration for analysis purposes.”

HOW DO YOU CALCULATE Grms CORRECTLY?! I KEEP GETTING SOMETHING WRONG. Closest I get is when I just take the square root of the area, but its off by about 7% for some reason

## MIL-STD-810F - Part 2 Laboratory Tests

### 514.5 Vibration (pp. 227-273)

Purpose: 1) Develop material to function in and withstand the vibration exposure of a life cycle and 2) verify that material will function in and withstand the vibration exposure of a life cycle.

Good quote: “Vibration results in dynamic deflections of and within materiel. These dynamic deflections and associated velocities and accelerations may cause or contribute to structural fatigue and mechanical wear of structures, assemblies, and parts. In addition, dynamic deflections may result in impacting of elements and/or disruption of function.”

Generally defines similar provisions as ASTM D4169, but in a military context.

While ASTM only requires a “pass/fail” metric for results, this document requires more thorough analysis of results. “It is necessary to relate the failure to the dynamic response of the materiel to the dynamic environment. Thus, include in failure analyses a determination of resonant mode shapes, frequencies, damping values and dynamic strain distributions in addition to the usual material properties, crack initiation locations, etc.”

## MIL-STD-2073-1

### Packaging and Marking Guide for DoD

## MSFC-STD-3676-A

### Development of Vibro-Acoustic and Shock Design and Test Criteria

## NASA-HDBK-7008

### Spacecraft Dynamic Environments Testing

# Structural Health Monitoring

## Development of a Smart-device-based Vibration-measurement System: Effectiveness examination and Application cases to existing structure

### Authors: Ashish Shrestha, Ji Dang, and Xin Wang

Structural Control Health Monitoring – 2018