Weapons Verification Summary

1.1 Introduction

- Discretization error is often a significant component of simulation result uncertainty. Therefore, quantification of discretization error (solution verification) is vitally important when an exact/analytical solution is not available.

- Many discretization error estimation methods use an “ansatz,” i.e., an assumed form for the discretization error.

- The ansatz assumption is justified by Richardson extrapolation based on a series expansion of the error in a mesh scale parameter.

- Extrapolation-based error estimators require that the solutions for various meshes are within the “asymptotic region.”

- Roache (1998) used a generalized Richardson extrapolation to define the Grid Convergence Index (GCI) and showed that the GCI is proportional to Richardson-extrapolated error of the fine grid solution using a factor of safety. Unfortunately, (1) there is no guarantee that all analyses can justify using a common factor of safety and (2) the GCI is not applicable when error does not converge monotonically.

- Other authors have shown that fitting splines to functions of error versus cell size is shown to give more accurate error estimates than a single-term ansatz for some cases.

- Some authors have addressed both monotonic and oscillatory convergence by taking the absolute value of the error.

- Tippets et al. (2007) showed that an optimization solver to minimize the ansatz fit residuals for CFD allowed for use of four or more meshes. Using four or more meshes provides information on the quality of fit and indicates the validity of the asymptotic assumption.

- Approaches using the Method of Nearby Problems (MNP) or the Method of Manufactured Solutions (MMS) have been used for mesh convergence, but both have difficulties: (1) implementing MNP or MMS in many simulation codes would be difficult/impossible, (2) not well suited to models with nonlinearities (contact, materials, shock fronts, etcetera), (3) error estimates are only truly applicable to the nearby problem, not the original. Thus, the modeling error magnitude is still very uncertain.

- Hemez (2007) developed a variation on Richardson extrapolation using “Functional Convergence” methods in which simulation outputs are projected onto orthogonal components instead of analyzing convergence on the data directly. The components are derived with a Singular Value Decomposition (SVD) of a data matrix.

- In this work, the error absolute value is assumed to converge exponentially in the mesh size and makes use of the optimization problem from Tippets et al. (2007).

1.2 Calculation Verification Approach

1.2.1 Error Ansatz Solution

Below is a typical error ansatz:

y-yi=A∆xi∆xrq.

Where, y^ is the extrapolated field value, yi is the field value given by a mesh i with mesh size ∆xi, ∆xr is a reference mesh size, A is a prefactor coefficient (equal to the error on mesh size r), and q is the rate of convergence. The [.] functional operator is an Lp norm with an integral approximation and domain that is defined by the ansatz implementation. An analysis using this equation would have three unknowns (y^, A, and q) which would require solution from at least three meshes. A minimization problem could be posed with the following cost function:

f2=i=1Ny-yi-A∆xi∆xrq2

N is equal to the number of meshes used. The fit error will be zero when N=3 and greater than zero with N>3.

Parameter Initialization

Unfortunately, the previously proposed cost function has discontinuous derivatives and (usually) local minima. These problems can cause difficulty for optimization solvers and the final solution may be very sensitive to initial values of the unknown parameters. This section describes a semi-analytical solution for the case of three meshes to overcome these issues. First, the first equation is modified:

y-yi=siA∆xi∆xrq

This modification introduces the sign of the error on mesh i in which si is either 1 or -1. Defining R as the following ration for a coarse (c), medium (m), and fine mesh (f) leads to an implicit solution that satisfies the equation if ∆xf/∆xm= ∆xm/∆xc

R=yf-ymym-yc=1-sfsm∆xf∆xmqsmsc∆xf∆xm-q-1

This equation becomes quadratic in (∆xm/∆xc)q and may be solved explicitly for q. Otherwise, a one-dimensional optimization procedure produces the correct value. Once q is known, A and y follow easily:

A=∆xrqyi-yjsj∆xjq-si∆xiq

y=yi+siA∆xi∆xrq

When only three meshes are used, this method produces ansatz parameters directly. For four or more solutions, this method can be used to generate initial values for an optimization solver.

Since multiple solutions for ansatz parameters exist for 4 or more meshes, different intervals of convergence exist (see paper for more description). Since the choice of solutions may be somewhat arbitrary, an alternative approach would be to include all possible convergence solutions in an expression for error range.

1.2.2. Functional Convergence

Time Histories

Field Variables

1.2.3. Metrics

Bootstrap and Residuals

Normalization and Convergent Fractions

1.3. Baseline Model Motivation